# SIMATS ENGINEERING

ASSESSMENT TOOL 3

# MCQ Test

## COURSE NAME: Computer Networks	COURSE CODE:	CSA07

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 20%

## QUESTIONS:20	Total Marks:20

- Which TCP mechanism reduces the congestion window by half upon detecting packet loss via duplicate ACKs?

  - Slow Start

  - Fast Retransmit

  - Fast Recovery

  - Tahoe Reset (BL4)(1)

- A TCP sender has CWND = 8 MSS and SSTHRESH = 16. After one RTT with no loss, CWND becomes:

  - 9 MSS

  - 16 MSS

  - 12 MSS

  - 8 MSS (BL4)(1)

- Which field in the TCP header is used to implement flow control at the receiver side?

  - Sequence number

  - Acknowledgement number

  - Window size

  - Urgent pointer (BL4)(1)

- Nagle's algorithm causes increased latency primarily because it:

  - Disables ACK piggybacking

  - Buffers small segments until ACK arrives

  - Reduces CWND on every packet

  - Increases SYN backlog (BL4)(1)

- In a high-latency satellite link (600 ms RTT), TCP throughput is primarily limited by:

  - MTU size

  - CWND growth rate relative to RTT

  - DNS resolution time

  - UDP competition (BL4)(1)

- UDP is preferred over TCP for DNS queries primarily because:

  - UDP supports fragmentation

  - DNS responses are small and connection overhead is unnecessary

  - TCP cannot use port 53

  - UDP has better error correction (BL4)(1)

- A UDP-based streaming application experiences 15% packet loss. The best application-layer mitigation is:

  - Increase UDP buffer size

  - Switch to TCP

  - Implement Forward Error Correction (FEC)

  - Reduce frame rate to 1 fps (BL4)(1)

- What is the maximum UDP payload size without IP-level fragmentation on a standard Ethernet link (MTU = 1500 bytes)?

  - 1480 bytes

  - 1472 bytes

  - 1460 bytes

  - 1500 bytes (BL4)(1)

- QUIC protocol uses UDP as its transport. The key advantage over TCP-based TLS is:

  - Eliminating IP header overhead

  - 0-RTT or 1-RTT connection setup

  - Removing encryption

  - Native multicast support (BL4)(1)

- HTTP/1.1 persistent connections reduce latency by:

  - Enabling UDP fallback

  - Reusing the same TCP connection for multiple requests

  - Compressing HTTP headers

  - Sending requests without waiting for DNS (BL4)(1)

- Head-of-line blocking in HTTP/1.1 occurs when:

  - The DNS server is slow

  - A response blocks subsequent responses on the same connection

  - TCP CWND drops to zero

  - The server rejects pipelined requests (BL4)(1)

- HTTP/2 multiplexing solves HOL blocking at the HTTP layer but not at the:

  - Application layer

  - DNS layer

  - TCP layer

  - IP layer (BL4)(1)

- Which HTTP status code indicates that a resource has permanently moved and browsers should update bookmarks?

  - 301

  - 302

  - 304

  - 307 (BL4)(1)

- In HTTP caching, the Cache-Control: no-cache directive means:

  - Never cache this resource

  - Validate with the server before using a cached copy

  - Cache for 0 seconds

  - Disable all caching headers (BL4)(1)

- Which DNS record type maps a domain name to an IPv6 address?

  - A

  - CNAME

  - AAAA

  - MX (BL4)(1)

- A DNS resolver switches from UDP to TCP when:

  - The query type is MX

  - The response is truncated (TC bit set)

  - TTL exceeds 3600 s

  - The authoritative server is unreachable (BL4)(1)

- Which attack exploits the UDP-based DNS protocol by sending forged responses with a matching transaction ID?

  - ARP poisoning

  - DNS cache poisoning

  - SYN flooding

  - BGP hijacking (BL4)(1)

- Setting a very low TTL (e.g., 30 s) for a DNS record primarily increases:

  - Cache hit ratio

  - Recursive resolver query load

  - DNSSEC signature validity

  - Zone transfer frequency (BL4)(1)

- A user browses to a new website. The correct sequence of protocol interactions for the first byte of the page is:

  - HTTP → TCP → DNS

  - DNS → TCP handshake → HTTP GET

  - TCP → DNS → HTTP

  - DNS → HTTP → TCP (BL4)(1)

- Under 20% packet loss, which protocol combination gives the best

performance for a real-time multiplayer game?

  - TCP for game state, HTTP for assets

  - UDP with application-layer sequencing for game state, HTTP/3 for assets

  - HTTP/2 for all traffic

  - TCP with Nagle enabled for all traffic (BL4)(1)

Code of Conduct:

I Dinesh Bharadwaj(192572028) (Name / Reg No) certify that this submission is my

original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

## RUBRICS FOR CALCULATION:

| Criteria | Weightag e (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Conceptual Understandin g | 30% | Demonstrates thorough understandin g; answers all questions accurately | Shows good understandin g with few errors | Partial understandin g; several incorrect responses | Lacks understandin g; majority of answers incorrect |
| Accuracy of Answers | 25% | All answers are correct | Most answers are correct with minor mistakes | Some correct answers; frequent errors | Mostly incorrect answers |
| Application | 20% | Correctly | Applies | Limited ability | Unable to |

| of Knowledge |  | applies concepts to problem-based questions | concepts with minor errors | to apply concepts | apply concepts |
| --- | --- | --- | --- | --- | --- |
| Time Management | 15% | Completes quiz well within time efficiently | Completes on time with minor delays | Requires extra time or rushes through questions | Unable to complete within given time |
| Question In terpretation | 10% | Interprets all questions correctly and responds appropriately | Misinterprets a few questions | Often misinterprets questions | Unable to understand questions |

# ANSWERS

# 1.C

# 2.B

# 3.C

# 4.B

# 5.B

# 6.B

# 7.C

# 8.B

# 9.B

# 10.B

# 11.B

# 12.C

# 13.B

# 14.C

# 15.C

# 16.B

# 17.B

# 18.B

# 19.B

# 20.B